# Scanner Setup

This Document explains the setup of a Barcode Scanner.

It is assumed that the Barcode Scanner hardware has been install on the POS terminal and verified to work.

## Overview

The Barcode Scanner connects to a Com Port on the POS Terminal. Com ports 1 through 5 maybe used. The terminal records hold the configuration data. The POS Edit forms are used to change the terminal records and configure the POS Terminal functionality. There are two basic uses for the Barcode Scanner. Reading purchase items Barcode for sale and reading barcode for SKU data entry. This document will cover setting the Barcode Scanner manufacturers (type), the communications port used, whether SKU editing is allowed at the POS terminal, and SKU edit method.

## Getting Started

On the Terminal Server window, select the File pull down menu, Select POS Data Edit. The following dialog should appear. See figure 1.

Figure 1

Use the arrow keys or tab key to place the cursor in the terminal Parameters field PS3TL, Then press the down page key.

The following Dialog should appear. See figure 2.

## Scanner Type

Use the arrow keys or tab to move the cursor to the Terminal # field, enter the Terminal Number for which record you want to edit, then Press the "ESC" key to retrieve the record. This will be the POS terminal you are connecting to the Barcode Scanner.

Figure 2

In this Dialog (figure 2) find the field labeled "Sn". Enter the Scanner type in this field.

In this example the type is 4, this will work for Specrtol Physics. To see a list of available types press the "F8" key while the cursor is in the "Sn" field and select from the list.

When you are done Press the "ESC" key to change and save the record.

press the "Page Up" key to return to the main menu dialog (figure 1).

## Setting the Com Port to Use

Tab or us the arrow keys to place the cursor in the terminal Parameters field PSTL2.

Then press the "Page Down" Key.

The following Dialog should appear (figure 3).

Figure 3

Use the arrow keys or tab to move the cursor to the Terminal # field, enter the Terminal Number for which record you want to edit, then Press the "ESC" key to retrieve the record. This will be the POS terminal you are connecting to the customer display.

Tab or us the arrow keys to place the cursor in the field labeled "Set Scanner COM Port #".

Enter the Com port the Customer Display is attached to.

In this example the Com port 1 has been chosen. This is the physical port on the POS Terminal where the Barcode Scanner is attached.

There is no help for this field. Pressing the "F8" key will do nothing.

## Enabling Keyboard for Edit

If you have a Keyboard attached to the POS terminal which makes SKU edit functions easier, you should enable the it's use. If you do not have a keyboard, you should disable this feature.

To enable or disable the use of the Keyboard, Use the arrow keys or tab to move the cursor to the field labeled "Enable Use FULL TRAVEL keyboard-SKU". Enter a "1" to Enable or a "0" to Disable.

When you are done Press the "ESC" key to change and save the record.

That is all that is required to setup the Port and Type for basic operation of the Barcode Scanner. However if you would like to be able to add Items not on file at the POS terminal, got to the next section "Enable SKU Edit".

If you are done editing records you can now exit the dialog by pressing the "END" key, or to edit another POS Terminals record, Press the "Page Up" key to return to the main menu dialog, page 1.

## Enable SKU Edit

Press the "Page Up" key to return to the main menu dialog, page 1.

Tab or us the arrow keys to place the cursor in the terminal Parameters field PSTL1.

Then press the "Page Down" Key.

The following Dialog should appear (figure 4).

Figure 4

Use the arrow keys or tab to move the cursor to the field labeled "Enable SKU Edit". Enter a "1" to Enable Editing at the POS terminal or a "0" to Disable.

When you are done Press the "ESC" key to change and save the record.

If you are done editing records you can now exit the dialog by pressing the "END" key, or to edit another POS Terminals record, Press the "Page Up" key to return to the main menu dialog, page 1.

## Using the Edit features at the POS terminal

When a Barcode is scanned the information is sent to the terminal server to retrieve information about the product from the SKU database. If data about the product exists, it is returned to the POS terminal for use. If no matching data is found, a dialog box will appear saying the item is not in the database. See  figure 5.

Figure 5

With a Keyboard

If you have enabled SKU editing at the POS terminal, you will see one of two different dialogs appear. If you have a keyboard attached and has been enabled, you will see the following dialog. See figure 6. If you do not have a keyboard see figure 7.

In this dialog you can enter the product information and apply this to the database. The next time the product is scanned, this information from the database will be retrieved.

Figure 6

If you do not have a keyboard or if it is not enabled you can still enter the product information and apply this to the database. Simply type the information on the provided keyboard. The upper left window or box is the description of the required information. The information typed will appear in the window or box below the description. Continue entering information as required until all information has been input.

The next time the product is scanned, information from the database will be retrieved.

Note the greater than and equal sign is the backspace key and the blank key to the right is the space bar.

Figure 7