## The Desktop shortcut

The above shortcut is created by browsing for the videoconsole program and right clicking on the program. Then Select  "Send to desktop ( create shortcut )". This will place a shortcut on the desktop like the one shown above. You can change the name of the short cut to anything that would help you remember its function. Such as "Start Video 1".

## The Shortcut Properties

The Target line in the above dialog contains parameter information required to operate properly.

Your shortcut will not contain the parameter required to operate. You will need to add these.

The Target consists of the following information.

Executable path

Name of the terminal server computer

Terminal ID

Data ID

Bump Bar COM Port

Video ID

The Exacutable path is normally the "c:\pos". This is where the executable program resides. With the executable this would be "c:\pos\videoconsole.exe".  this should have been added when the shortcut was created.

The following parameters are added with a space before each one.

In the dialog above the first parameter is "svr" you will replace this with the computer name of the terminal server.

The second parameter is the terminal ID. There must be a unique terminal record for each vidioconsole. In this example 22 is the terminal ID.

WARNING NOTE: The software key can affect the number of terminals available for use.

The third Parameter is the DATA: parameter which is an identifier.

The Forth parameter is the COM port for the Bump Bar associated with this video console.

The last parameter is the Video port to on which to send the image. The videoconsole program can use the extended desktop of a multi video controler system. Each controller and therefore each monitor connected is assigned an arrea of the desktop. VIDEO:1 in the example is actually the second monitor attached.

NOTE: these paramerters must be added in the order given.

Example:

"C:\POS\VIDEOCONSOLE.EXE" MYSERVERSNAME 22 DATA:1 PORT:1 VIDEO:1

## Routing PLU's

The videoconsole program uses a test file "vplulist.doc" to route  PLU's to the proper videoconsole.

The file is a simple text file which can be created using  "Notepad" or the msdos "Edit" program. The following in a sample from a "vplulist.doc" file.

1,10000,19999

1,22000,22499

2,30000,33000

2,10000,19999

The fist column before the comma is the video console to route the following range of PLU's.

The next two fields are the low PLU range and the high range.

In the example the only PLU's from 10000 to 19999 and the PLU's from 22000 to 22499 will be displayed on videoconsole 1. Only PLU's from 30000 to 33000 and the PLU's from 10000 to 19999 will be displayed on videoconsole 2.